SIGNALFIX

V2.1

Formal Mathematical Specification — Corrected & Augmented

Document Class: Technical Architecture Specification

Filter Class: Linear Adaptive Kalman Filter, Single Sensor, Scalar Measurement

Version: 2.1 — Post-Audit Corrected Draft

Supersedes: SignalFix V2.0 Pre-Implementation Audit Draft

| Audit Basis | Control Systems Research Review — Graduate Level |
| --- | --- |
| Critical Fixes Applied | 4 of 4 |
| High Severity Fixes Applied | 4 of 4 |
| Structural Upgrades Added | 10 of 10 |
| New Mechanisms | Bias State, Whiteness Monitor, Calibrated Confidence |

# 1. System Assumptions

## 1.1 Linear State-Space Assumptions

The system assumes a discrete-time linear state-space model of the form:

x_k = F_k x_{k-1} + w_k z_k = H x_k + v_k

The following linearity conditions hold strictly:

- The state transition from x_{k-1} to x_k is an affine (linear) function of the previous state

- The measurement z_k is a linear projection of the true state

- Process noise w_k and measurement noise v_k are zero-mean, uncorrelated, white Gaussian sequences

- w_k ~ N(0, Q_k) and v_k ~ N(0, R_k)

These assumptions collectively mean the optimal estimator is the Kalman filter itself — no approximation is introduced. Any deviation from linearity or Gaussianity degrades optimality; see Section 14 for explicit failure boundaries.

## 1.2 Constant Velocity Model

The physical model assumes the tracked quantity evolves under constant velocity dynamics with small stochastic perturbations:

p_k = p_{k-1} + p-dot_{k-1} * dt + epsilon_p p-dot_k = p-dot_{k-1} + epsilon_{p-dot}

This model is valid when: (1) true dynamics contain no sustained acceleration; (2) acceleration, when present, is brief and captured by the process noise model; (3) sampling rate dt^{-1} is sufficiently high relative to the bandwidth of true dynamics.

## 1.3 Observability Assumptions

The observability matrix O for the system pair (H, F) is:

O = [H; HF] = [[1, 0]; [1, dt]]

rank(O) = 2 for all dt > 0, confirming full observability. Both position and velocity are recoverable from position measurements alone.

| NEW | V2.1 ADDITION — Extended State Observability: With the bias state added (Section 2.1), the state is [p, p-dot, b]^T and H = [1, 0, 1]. The extended observability matrix O_ext has rank 3 for dt > 0 provided q_b > 0, confirming the bias state remains identifiable from the innovation sequence. |
| --- | --- |

Observability degrades when: dt -> 0 (numerical rank collapse); measurements arrive at perfectly constant intervals with correlated noise (coloured noise violates white noise assumption); or the bias random walk variance q_b = 0 (bias becomes unidentifiable).

## 1.4 Stated Limitations

| Limitation | Consequence |
| --- | --- |
| Linearity required | Nonlinear dynamics invalidate optimality — use EKF or UKF (Section 14) |
| Gaussian noise required | Non-Gaussian tails cause NIS miscalibration |
| White noise required | Correlated noise inflates covariance incorrectly — whiteness monitor (Section 5C) detects this |
| Single sensor | No redundancy; no cross-validation between sensors |
| Constant velocity only | Manoeuvring targets cause systematic innovation growth |
| Scalar dt timing jitter | Timing error appears as process noise and contaminates adaptive R; dt_max ceiling (Step 1) mitigates |

# 2. State Definition

## 2.1 State Vector

| FIX | V2.0 CRITICAL FIX: Bias state b_k added. V2.0 had no bias state, making sensor DC offset an undetectable silent failure. A converged filter absorbs bias into the position estimate with high confidence — the most dangerous failure mode. The extended state is now [p_k, p-dot_k, b_k]^T. |
| --- | --- |

The state vector is extended to include a sensor bias term:

x_k = [p_k, p-dot_k, b_k]^T in R^{3x1}

where p_k is position (directly observed), p-dot_k is velocity (latent, inferred from position sequence), and b_k is sensor bias (latent, inferred from persistent innovation mean).

## 2.2 State Transition Matrix

The transition matrix expands to 3x3 to accommodate the bias random walk:

F(dt) = [[1, dt, 0], [0, 1, 0], [0, 0, 1]] in R^{3x3}

The bias state follows a random walk (third row is identity), driven by process noise q_b. This is the standard augmented constant-velocity-plus-bias model.

## 2.3 Measurement Matrix

| FIX | V2.0 FIX: H extended to account for bias in measurement. In V2.0, H = [1, 0] ignored bias contribution to z_k. The correct model is z_k = p_k + b_k + v_k, giving H = [1, 0, 1]. |
| --- | --- |

H = [1, 0, 1] in R^{1x3}

Position and bias are both observed through the scalar measurement. Velocity remains entirely latent.

## 2.4 Dimensional Summary

| Symbol | Dimension | Meaning |
| --- | --- | --- |
| x_k | 3 x 1 | State vector [position, velocity, bias] |
| F | 3 x 3 | State transition matrix (extended) |
| P_k | 3 x 3 | Error covariance (extended) |
| Q_k | 3 x 3 | Process noise covariance (extended) |
| H | 1 x 3 | Measurement matrix (extended, includes bias) |
| R_k | 1 x 1 | Measurement noise variance (scalar) |
| K_k | 3 x 1 | Kalman gain (extended) |
| z_k | scalar | Raw measurement |
| y_k | scalar | Innovation |
| S_k | scalar | Innovation covariance |
| NIS_k | scalar | Normalised innovation squared |
| b_k | scalar | Sensor bias state |

# 3. Prediction Step (Time Update)

## 3.1 State Prediction

x-hat_{k¦k-1} = F_k * x-hat_{k-1¦k-1}

Propagates the previous posterior estimate forward through the deterministic dynamics. With the extended state, this includes the bias prediction b_{k|k-1} = b_{k-1|k-1} (bias held constant under random walk model).

## 3.2 Covariance Prediction

P_{k¦k-1} = F_k * P_{k-1¦k-1} * F_k^T + Q*(dt)

| FIX | V2.0 FIX: Q*(dt) is now the single-pass scaled Q applied directly here. The V2.0 two-pass design (base Q in Step 3, scaled Q recomputed in Step 7 using NIS from Step 5) created a mathematical inconsistency: the NIS driving Q adaptation was computed from a different P than the one used in the final gain. V2.1 uses lagged NIS (NIS_{k-1}) from the previous step to scale Q before prediction, making the adaptive Q causal and formally consistent. |
| --- | --- |

## 3.3 Process Noise Matrix — Van Loan Discretisation (Extended)

For the extended 3-state model, the process noise matrix is block-diagonal. The position-velocity block uses the Van Loan discretisation; the bias block uses a separate intensity q_b:

Q(dt) = q * [[dt^3/3, dt^2/2, 0], [dt^2/2, dt, 0], [0, 0, 0]] + q_b * [[0, 0, 0], [0, 0, 0], [0, 0, 1]]

Physical justification of each term:

- q * dt^3/3 — Position variance: random acceleration integrates twice, variance grows as dt^3

- q * dt — Velocity variance: random acceleration integrates once, variance grows linearly with dt

- q * dt^2/2 — Cross-correlation: position and velocity uncertainties are coupled through the same acceleration event; omitting this breaks the covariance structure

- q_b * 1 — Bias variance: the bias random walk injects fresh uncertainty each step, independent of dt in the zero-order hold sense; q_b controls how quickly bias can drift

The complete Q(dt) matrix is positive semi-definite for all dt >= 0 and positive definite for dt > 0, guaranteeing P_{k|k-1} remains positive definite given P_{k-1|k-1} > 0.

# 4. Innovation Step

## 4.1 Innovation

y_k = z_k - H * x-hat_{k¦k-1}

The innovation is the discrepancy between the actual measurement z_k and the predicted measurement H * x-hat_{k|k-1} = p-hat_{k|k-1} + b-hat_{k|k-1}. With the bias state included, a persistent sensor offset now drives b-hat toward the true bias rather than contaminating p-hat. Under correct model assumptions, y_k ~ N(0, S_k).

## 4.2 Innovation Covariance

S_k = H * P_{k¦k-1} * H^T + R_k

The term H P H^T projects prior state uncertainty (now 3x3) into scalar measurement space. With H = [1, 0, 1], this projection includes position-bias cross-covariance: H P H^T = P[0,0] + P[0,2] + P[2,0] + P[2,2]. The term R_k is the independent measurement noise. S_k is always a positive scalar.

## 4.3 Normalised Innovation Squared (NIS)

NIS_k = y_k^T * S_k^{-1} * y_k = y_k^2 / S_k

Under correct model assumptions, NIS_k ~ chi^2(1), with mean 1 and variance 2. This distribution is the central diagnostic instrument: NIS ~ 1 indicates well-calibrated filter; NIS >> 1 indicates overconfidence (true uncertainty exceeds modelled); NIS << 1 indicates underconfidence.

NIS drives every adaptive mechanism in SignalFix V2.1. The lagged value NIS_{k-1} is used for adaptive Q scaling (Section 6) to maintain causal consistency.

# 5. Innovation Sequence Monitoring

| NEW | V2.1 NEW SECTION — Innovation Whiteness Monitor and Calibrated Confidence. Added to detect silent failure modes that NIS magnitude alone cannot catch: sensor bias (now partially handled by bias state, but residual drift still possible), correlated process noise, and unmodelled dynamics. These mechanisms were absent in V2.0. |
| --- | --- |

## 5A. Innovation Whiteness Monitor

Under correct model assumptions, the innovation sequence {y_k} is a zero-mean white Gaussian process — successive innovations must be uncorrelated. Systematic correlation indicates model mismatch.

Define the lag-1 sample autocorrelation of the normalised innovation:

rho_k = (y_k * y_{k-1}) / S_k

Under correct model: E[rho_k] = 0. A running exponential average tracks this:

rho-bar_k = lambda_w * rho-bar_{k-1} + (1 - lambda_w) * rho_k

Whiteness alarm condition:

If ¦rho-bar_k¦ > T_white => raise WHITENESS_ALARM

- T_white is set from the chi-squared distribution at the desired false alarm rate

- Persistent |rho-bar| > T_white indicates: correlated noise (violating white noise assumption), unmodelled dynamics such as sustained acceleration, or residual bias not captured by the bias state

- WHITENESS_ALARM does not trigger SAFE_MODE directly; it flags the output as model-inconsistent for downstream consumers

- lambda_w in (0,1) controls the whiteness monitor memory, tuned independently from the adaptive R lambda

## 5B. Mehra-Corrected Adaptive R

| FIX | V2.0 CRITICAL FIX: The V2.0 adaptive R estimator tracked y_k^2 directly and used it as R_k. This conflates total innovation variance S_k with measurement noise variance R_k. The innovation y_k contains contributions from both H P_{k¦k-1} H^T (state uncertainty) and R_k. Mehra (1970) showed the correct estimator must subtract the state uncertainty projection. Without this correction, R_k is systematically overestimated whenever P is large, creating a positive feedback loop: inflated P -> overestimated R -> reduced gain -> further P inflation. |
| --- | --- |

Define the windowed sample innovation covariance over the last W steps:

C_{yy,k} = (1/W) * sum_{j=k-W+1}^{k} y_j^2

The Mehra (1970) corrected R estimator subtracts the state uncertainty projection:

R-hat_k = C_{yy,k} - H * P_{k¦k-1} * H^T

Apply bounds to prevent numerical collapse:

R_k = clamp(R-hat_k, R_min, R_max)

where clamp(v, a, b) = max(a, min(v, b)), R_min > 0, R_max < infinity.

The Mehra estimator correctly isolates R_k from the total innovation variance. When P is large (startup, post-inflation), the H P H^T subtraction prevents R from being artificially inflated. The clamp ensures R_k >= R_min > 0 even when the subtraction yields a negative value (possible in finite windows), preventing S_k from collapsing to zero.

Stability condition: E[y_k^2] < infinity and W >= 1. The clamp independently guarantees R_k stability regardless of the estimator behaviour.

## 5C. Exponential-Smoothed R (Auxiliary)

As an auxiliary estimate for rapid noise tracking, the exponential smoother from V2.0 is retained but used only as a secondary signal, not as the primary R estimate:

sigma-hat^2_k = lambda * sigma-hat^2_{k-1} + (1 - lambda) * y_k^2

This secondary estimate is used to detect abrupt noise onset (fast timescale, lambda ~ 0.9) while the Mehra estimator provides the statistically correct long-term R. The final R_k is taken from the Mehra estimator; sigma-hat^2_k feeds the noise state diagnostic only.

# 6. Adaptive Process Noise Model

## 6.1 Motivation and Identifiability Constraint

| AUDIT | V2.0 AUDIT NOTE — Identifiability: Simultaneously adapting Q and R from a scalar innovation sequence is an unidentifiable problem. Given only {y_k}, there exists a continuum of (Q, R) pairs consistent with the observed innovation covariance. V2.1 addresses this by: (1) using the Mehra-corrected R estimator (Section 5B) which provides a more independent R estimate; (2) restricting Q adaptation to a bounded, one-step-lagged scaling that cannot compound across steps; (3) using the whiteness monitor (Section 5A) as an independent diagnostic that provides information orthogonal to NIS magnitude. Full identifiability requires external constraints or prior information. The partial identifiability mitigation implemented here is the maximum achievable without a second sensor or structural constraints on Q/R ratio. |
| --- | --- |

Adaptive Q addresses model smugness: the tendency of a converged filter to assign very small P_{k|k-1}, making it resistant to true state changes. The Q scaling is explicitly heuristic and is documented as such.

## 6.2 Lagged NIS-Driven Q Scaling

| FIX | V2.0 CRITICAL FIX: V2.0 computed NIS_k^nom in Step 5 using P_{k¦k-1} from Step 3 (with Q_base), then recomputed P_{k¦k-1} in Step 7 with Q* derived from that NIS. This created a two-pass inconsistency: the diagnostic was computed from a different P than the one used in the final gain. V2.1 uses NIS_{k-1} (previous step) to scale Q_k before prediction. This makes adaptive Q causal: Q_k is determined before P_{k¦k-1} is computed, so there is no circularity. |
| --- | --- |

Define the scaling function using the previous step's NIS:

f(NIS_{k-1}) = 1 if NIS_{k-1} <= tau_low = 1 + alpha * (NIS_{k-1} - tau_low) if tau_low < NIS_{k-1} <= tau_high / (tau_high - tau_low) = 1 + alpha if NIS_{k-1} > tau_high

where tau_low is the NIS value below which no Q adaptation occurs; tau_high is the saturation point; alpha > 0 is the maximum additional scaling factor bounded by alpha_max.

The scaled Q applied in prediction:

Q*(dt) = Q(dt) * f(NIS_{k-1})

f(NIS_{k-1}) in [1, 1 + alpha_max]. The base Q(dt) is recomputed fresh each step from the fixed parameter q and current dt — no accumulation of scale factors occurs across steps. The one-step lag ensures the total inflation bound per step is computable in advance.

# 7. Measurement Gating

## 7.1 Gating Decision Rule

delta_k = REJECT if NIS_k > T_gate delta_k = ACCEPT if NIS_k <= T_gate

## 7.2 Chi-Square Basis for Threshold Selection

Since NIS_k ~ chi^2(1) under correct model assumptions, T_gate is selected from the chi-squared CDF. Standard thresholds for 1 degree of freedom:

| Confidence Level | T_gate for chi^2(1) | False Rejection Rate |
| --- | --- | --- |
| 95% | 3.84 | 5 x 10^{-2} |
| 99% | 6.63 | 1 x 10^{-2} |
| 99.9% | 10.83 | 1 x 10^{-3} |
| 99.99% | 15.14 | 1 x 10^{-4} |
| Default V2.1 | 25.0 | 4.4 x 10^{-7} |

## 7.3 Consequence of Rejection

- State estimate carries forward: x-hat_{k|k} = x-hat_{k|k-1}

- Covariance carries forward: P_{k|k} = P_{k|k-1}

- Rejection counter increments: c_k = c_{k-1} + 1

- On acceptance: c_k = 0, update proceeds normally

# 8. Covariance Update — Joseph Form

## 8.1 Kalman Gain

K_k = P_{k¦k-1} * H^T * S_k^{-1} in R^{3x1}

## 8.2 State Update

x-hat_{k¦k} = x-hat_{k¦k-1} + K_k * y_k

The position, velocity, and bias are all updated jointly. The bias update K_k[2] * y_k drives b-hat toward the true sensor offset over time.

## 8.3 Covariance Update — Joseph Form

P_{k¦k} = (I - K_k H) P_{k¦k-1} (I - K_k H)^T + K_k R_k K_k^T

The Joseph form is mandatory for numerical stability. The standard form P = (I - KH)P is algebraically equivalent at infinite precision but numerically catastrophic in finite arithmetic — floating-point cancellation destroys positive definiteness over thousands of iterations. The Joseph form is a congruence transformation that cannot produce negative diagonal entries from a positive definite input.

## 8.4 Symmetrisation

P_{k¦k} <- (1/2) * (P_{k¦k} + P_{k¦k}^T)

Applied after every predict and update step. Corrects sub-machine-epsilon asymmetries that accumulate over long runs.

## 8.5 Diagonal Floor

P_{k¦k}[i,i] <- max(P_{k¦k}[i,i], P_min) for all i

Prevents diagonal entries from reaching or crossing zero due to numerical underflow. Applied after symmetrisation at both predict and update.

# 9. Inflation Logic

## 9.1 Selective Diagonal Inflation

| FIX | V2.0 FIX: V2.0 inflated only position [0,0] and velocity [1,1] diagonals independently while leaving off-diagonals unchanged. This can break positive definiteness if the off-diagonal P[0,1] dominates. V2.1 verifies the positive definiteness condition after every inflation event and applies a symmetrised correction if violated. The velocity inflation rationale also clarified: beta_vel = 1 is the default (velocity not inflated), consistent with the stated physical rationale. beta_vel is retained as a parameter only for cases where velocity is known to be unreliable. |
| --- | --- |

Selective inflation expands position and bias uncertainty independently, preserving velocity confidence:

P_{k¦k-1}[0,0] <- P_{k¦k-1}[0,0] * beta_pos P_{k¦k-1}[2,2] <- P_{k¦k-1}[2,2] * beta_bias (velocity diagonal P[1,1] is not inflated by default: beta_vel = 1)

Post-inflation positive definiteness check: the leading minor condition must hold for the 3x3 matrix. If the determinant condition fails after inflation, a symmetrised full-matrix scaling is applied as fallback:

If det(P_inflated) <= 0: P_{k¦k-1} <- beta_pos * P_{k¦k-1} (full matrix scale)

## 9.2 Inflation Trigger Condition

Fire inflation if: c_k > 0 AND c_k mod N = 0 AND c_k <= M

Inflation fires every N consecutive rejections. The counter c_k is NOT reset upon inflation — it continues accumulating toward the hard-fail threshold M.

## 9.3 Corrected Divergence Prevention Bound

| FIX | V2.0 CRITICAL FIX: The V2.0 bound P_max <= P_0 * beta_max^{floor(M/N)} was incorrect because it omitted the multiplicative contribution of adaptive Q scaling. The correct bound accounts for both mechanisms. Since V2.1 uses lagged NIS Q scaling with no cross-step accumulation, the one-step maximum Q inflation factor is (1 + alpha_max). Over M rejection steps before SAFE_MODE, inflation fires floor(M/N) times and Q scaling applies at most M times. The correct computable bound is: |
| --- | --- |

P_max <= P_0 * beta_max^{floor(M/N)} * (1 + alpha_max)^M

This is finite and can be computed in advance for any parameter set. Parameter selection must verify this bound does not exceed the NaN recovery threshold alpha_init * I.

# 10. Mode Transition Logic

## 10.1 Mode Definitions

| Mode | Condition | Active Operations |
| --- | --- | --- |
| NORMAL | c_k < N | Full pipeline: predict, gate, inflate, update, whiteness monitor |
| DEGRADED | N <= c_k <= M | Full pipeline plus active inflation at every N-th rejection |
| SAFE_MODE | c_k > M | Prediction only; update suspended; whiteness monitor continues |

## 10.2 Transition Conditions

NORMAL -> DEGRADED: c_k >= N DEGRADED -> SAFE_MODE: c_k > M DEGRADED -> NORMAL: c_k = 0 (measurement accepted) SAFE_MODE -> NORMAL: absolute recovery condition (Section 10.3)

## 10.3 Corrected SAFE_MODE Recovery Condition

| FIX | V2.0 CRITICAL FIX: The V2.0 recovery condition (NIS_k <= T_gate) was mathematically vacuous in SAFE_MODE. During open-loop prediction, P_{k¦k-1} grows without bound, causing S_k -> infinity and NIS_k -> 0 regardless of sensor health. The condition was therefore always satisfied eventually, even for permanently dead sensors. The correct condition tests whether the measurement itself is plausible in absolute terms, independently of the inflated P. |
| --- | --- |

Recovery from SAFE_MODE requires n_recovery consecutive steps satisfying the absolute innovation magnitude test:

Recovery condition: ¦y_k¦ <= T_recover * sqrt(R_nominal) for n_recovery consecutive steps

where T_recover is a fixed multiplier (typically 3.0, corresponding to a 3-sigma test under nominal noise), and R_nominal is the fixed design-time measurement noise variance — not the adaptive R_k. This tests the measurement itself, not the filter's current uncertainty.

The recovery streak counter increments on each step satisfying the absolute condition and resets on any violation. Recovery to NORMAL mode triggers a P reset: P <- alpha_recovery * I where alpha_recovery is chosen to ensure the filter re-converges rapidly without the state jump that would occur from carrying forward the inflated P.

Constraint: M > N > 0, n_recovery >= 1, T_recover > 0, alpha_recovery <= alpha_init.

# 11. Confidence Metric — Calibrated

| FIX | V2.0 FIX: The V2.0 exponential-of-trace metric had no calibrated probabilistic interpretation. The thresholds (0.4, 0.8) were arbitrary. V2.1 replaces this with a genuine statistical confidence measure derived from the chi-squared distribution, providing thresholds with exact statistical meaning. |
| --- | --- |

## 11.1 Primary Confidence Metric — NIS p-value

Define the per-step confidence metric as the survival function of the chi-squared distribution evaluated at the current NIS:

gamma_k = 1 - F_{chi^2(1)}(NIS_k) = P(chi^2(1) >= NIS_k)

where F_{chi^2(1)} is the chi-squared CDF with 1 degree of freedom. This is a genuine probability with precise meaning: the probability of observing an innovation this extreme or more extreme under the current filter model. Thresholds now carry exact statistical interpretation:

| gamma_k range | Statistical Meaning | Interpretation |
| --- | --- | --- |
| gamma_k > 0.05 | NIS in the 95th percentile or below | Filter consistent with model — output trustworthy |
| 0.01 < gamma_k <= 0.05 | NIS between 95th and 99th percentile | Elevated stress — monitor; flag output |
| gamma_k <= 0.01 | NIS above 99th percentile | Model inconsistency detected — output suspect |
| gamma_k ~ 0 | NIS >> chi^2 expectation | Filter has lost consistency — treat as DEGRADED regardless of mode |

## 11.2 Secondary Metric — Covariance Trace (Retained)

The exponential-of-trace metric from V2.0 is retained as a secondary uncertainty indicator, not a confidence measure:

kappa_k = exp(-tr(P_{k¦k}) / sigma^2_ref)

kappa_k is explicitly NOT interpreted as a probability. It is a monotone uncertainty indicator used to detect P divergence (kappa_k -> 0) and over-convergence (kappa_k -> 1). sigma^2_ref is the expected steady-state trace under nominal conditions.

## 11.3 Combined Output

Both metrics are reported as distinct outputs with clearly labelled interpretations:

- gamma_k: statistical confidence (p-value) — has a rigorous probabilistic meaning

- kappa_k: uncertainty indicator — dimensionless, monotone, no probabilistic interpretation

- whiteness_alarm: boolean — flags innovation sequence correlation

# 12. Full Mathematical Flow — V2.1 Corrected Pipeline

## Step 1 — Clock and dt Ceiling

dt_k = t_k - t_{k-1} dt_k <- max(dt_k, dt_min) dt_k <- min(dt_k, dt_max) [NEW: ceiling prevents Q explosion at large gaps]

## Step 2 — Build Extended Transition Matrix

F_k = [[1, dt_k, 0], [0, 1, 0], [0, 0, 1]]

## Step 3 — Lagged Q Scaling and Prediction

Use NIS_{k-1} from the previous step (causal, no circularity):

f(NIS_{k-1}) = piecewise linear ramp in [1, 1 + alpha_max] Q*(dt_k) = Q(dt_k) * f(NIS_{k-1}) x-hat_{k¦k-1} = F_k * x-hat_{k-1¦k-1} P_{k¦k-1} = F_k * P_{k-1¦k-1} * F_k^T + Q*(dt_k) P_{k¦k-1} <- symmetrize(apply_floor(P_{k¦k-1}))

## Step 4 — Innovation

y_k = z_k - H * x-hat_{k¦k-1}

## Step 5 — Whiteness Monitor Update

rho_k = y_k * y_{k-1} / S_{k-1} [lag-1 normalised product] rho-bar_k = lambda_w * rho-bar_{k-1} + (1 - lambda_w) * rho_k whiteness_alarm_k = (¦rho-bar_k¦ > T_white)

## Step 6 — Mehra-Corrected Adaptive R

C_{yy,k} = (1/W) * sum_{j=k-W+1}^{k} y_j^2 R-hat_k = C_{yy,k} - H * P_{k¦k-1} * H^T R_k = clamp(R-hat_k, R_min, R_max) S_k = H * P_{k¦k-1} * H^T + R_k S_k <- max(S_k, epsilon)

## Step 7 — NIS Computation

NIS_k = y_k^2 / S_k gamma_k = 1 - F_{chi^2(1)}(NIS_k) [calibrated confidence] kappa_k = exp(-tr(P_{k¦k}) / sigma^2_ref) [uncertainty indicator]

## Step 8 — Mode Check (pre-gate)

If system_state = SAFE_MODE: check absolute recovery condition.

If ¦y_k¦ <= T_recover * sqrt(R_nominal): recovery_streak_k = recovery_streak_{k-1} + 1 Else: recovery_streak_k = 0 If recovery_streak_k >= n_recovery: system_state <- NORMAL P_{k¦k-1} <- alpha_recovery * I [P reset on recovery] Else: skip to Step 13 (prediction only)

## Step 9 — Gating

If NIS_k > T_gate: REJECT, c_k <- c_k + 1 Else: ACCEPT, c_k <- 0

## Step 10 — Selective Inflation

If c_k > 0 AND c_k mod N = 0 AND c_k <= M:

P_{k¦k-1}[0,0] <- P_{k¦k-1}[0,0] * beta_pos P_{k¦k-1}[2,2] <- P_{k¦k-1}[2,2] * beta_bias Verify det(P_{k¦k-1}) > 0; apply full-matrix fallback if not

## Step 11 — Mode Transition Check

c_k > M => system_state <- SAFE_MODE c_k >= N => system_state <- DEGRADED c_k = 0 => system_state <- NORMAL

## Step 12 — Update (Joseph Form, if ACCEPT and not SAFE_MODE)

K_k = P_{k¦k-1} * H^T / S_k x-hat_{k¦k} = x-hat_{k¦k-1} + K_k * y_k P_{k¦k} = (I - K_k H) P_{k¦k-1} (I - K_k H)^T + K_k R_k K_k^T P_{k¦k} <- symmetrize(apply_floor(P_{k¦k}))

If REJECT or SAFE_MODE:

x-hat_{k¦k} = x-hat_{k¦k-1} P_{k¦k} = P_{k¦k-1}

## Step 13 — NaN Guard

If any element of x-hat_{k¦k} is not finite, or any diagonal of P_{k¦k} is not in R_{>0}: x-hat_{k¦k} <- x-hat_{k¦k-1} P_{k¦k} <- alpha_init * I

## Step 14 — Output

Return the following at every step:

| Output | Type | Meaning |
| --- | --- | --- |
| x-hat_{k¦k} | 3-vector | Filtered state: [position, velocity, bias] |
| diag(P_{k¦k}) | 3-vector | Diagonal uncertainty for each state |
| NIS_k | scalar | Normalised innovation squared |
| R_k | scalar | Current adaptive measurement noise estimate |
| c_k | integer | Consecutive rejection count |
| system_state | enum | NORMAL / DEGRADED / SAFE_MODE |
| gamma_k | scalar in [0,1] | Statistical confidence (chi-squared p-value) |
| kappa_k | scalar in (0,1] | Covariance uncertainty indicator |
| whiteness_alarm_k | boolean | Innovation sequence correlation detected |

# 13. Stability Guarantees

## 13.1 NaN Prevention

NaN propagation is blocked at four independent layers: (1) dt_min and dt_max floors bound F and Q to finite matrices; (2) S_k >= epsilon > 0 prevents division by zero in NIS and gain computation; (3) R_k >= R_min > 0 ensures S_k >= H P H^T + R_min > 0; (4) the NaN guard in Step 13 detects any surviving NaN and resets to a large-uncertainty prior for rapid recovery.

## 13.2 Negative Variance Prevention

Positive definiteness of P_{k|k} is maintained by three redundant mechanisms: (a) Joseph form is a congruence transformation that cannot produce negative diagonal entries from a positive definite input; (b) the P_min diagonal floor enforces a hard lower bound after every step; (c) symmetrisation prevents asymmetric numerical drift. All three must fail simultaneously for negative variance to occur.

The post-inflation positive definiteness check (Step 10) adds a fourth mechanism specific to selective inflation.

## 13.3 Corrected Divergence Prevention

The filter cannot diverge to infinity because: (a) Q*(dt) = Q_base * f(NIS_{k-1}) with f bounded in [1, 1 + alpha_max], and Q_base -> 0 as dt -> 0 (the dt_max ceiling prevents the other extreme); (b) R_k >= R_min prevents gain collapse toward zero; (c) the corrected bounded inflation expression P_max <= P_0 * beta_max^{floor(M/N)} * (1 + alpha_max)^M is finite and computable; (d) SAFE_MODE with the corrected absolute recovery condition prevents runaway inflation compounding indefinitely.

## 13.4 Closed-Loop Adaptive Stability — Qualitative Argument

| AUDIT | AUDIT NOTE: A formal Lyapunov stability proof for the three coupled adaptive loops (adaptive R, adaptive Q, inflation/mode logic) is beyond the scope of this specification. The following is a qualitative bounded-input-bounded-output argument. A full stability proof would require a Lyapunov function for the composite nonlinear time-varying system. This is identified as a required extension before deployment in safety-critical applications. |
| --- | --- |

Qualitative BIBO argument: Each adaptive loop has bounded inputs and bounded outputs by construction. The adaptive R output R_k in [R_min, R_max]. The adaptive Q scale f(NIS) in [1, 1+alpha_max]. The inflation applies at most floor(M/N) times before SAFE_MODE terminates measurement processing. The mode transitions form a finite state machine with known entry and exit conditions. Under bounded measurement noise with finite second moment, the innovation sequence {y_k} has finite variance, which bounds C_{yy,k} and therefore R-hat_k. Combined with the clamp, R_k remains bounded. The Q scaling uses a lagged NIS that is itself bounded by the gating threshold (rejected measurements do not alter the filter state). The composite system is therefore bounded in each component. Interaction effects between loops are mitigated by the one-step lag in Q adaptation (preventing immediate feedback) and the independent Mehra R estimator (reducing the coupling between adaptive Q and adaptive R).

## 13.5 Conditions Where Divergence Remains Possible

- True dynamics are nonlinear: F no longer accurately propagates the state; use EKF or UKF (Section 14)

- True noise has infinite variance: R_max clamp locks the filter in near-open-loop operation

- dt_max ceiling violated in deployment: Q(dt) grows as dt^3; the ceiling is mandatory, not optional

- Bias drift faster than q_b permits: bias state will lag true bias, producing persistent elevated NIS that both adaptive mechanisms will partially absorb incorrectly

- Correlated noise with zero mean: whiteness alarm detects but the filter has no mechanism to correct for correlated noise structure; this remains a fundamental limitation of the linear Gaussian framework

# 14. Explicit Limitations

## 14.1 When EKF is Required

EKF is required when the state transition f(x) or measurement function h(x) is nonlinear. Examples: bearing-only tracking (nonlinear h), constant turn-rate models (nonlinear f), range-bearing sensors. EKF linearises via Jacobians at the current estimate; accuracy degrades when nonlinearities are severe.

## 14.2 When UKF is Required

UKF is required when nonlinearities are strong enough that first-order linearisation introduces significant error, or when the posterior is markedly non-Gaussian after propagation. UKF propagates sigma points through exact nonlinear functions, capturing mean and covariance to second order without Jacobians.

## 14.3 Residual Bias Identifiability Limit

The bias state b_k addresses slowly-drifting DC offset. The filter cannot distinguish between: a step change in true position and a step change in sensor bias, during the transient before the bias estimate converges. During this transient, the innovation mean will be non-zero. The whiteness monitor will alarm. The adaptive R will increase. The filter's response to these indistinguishable root causes is identical: increase uncertainty and reduce gain. This is physically correct but produces a temporarily degraded position estimate during any large, rapid bias change.

## 14.4 When Observability Fails

- dt = 0 for multiple consecutive steps: velocity unobservable, rank collapse in extended O

- q_b = 0: bias state is a random constant, not identifiable from finite data without additional structure

- Measurements far slower than dynamics bandwidth: velocity estimate effectively unconstrained between updates

- SAFE_MODE open-loop: state uncertainty grows at O(dt^3) for position and O(dt) for velocity; the bias state uncertainty grows at O(q_b) per step regardless of dt

## 14.5 Simultaneous Q-R Identifiability — Residual Limitation

As noted in Section 6.1, simultaneous adaptation of Q and R from a scalar innovation sequence is fundamentally unidentifiable without additional constraints. The Mehra correction (Section 5B) reduces but does not eliminate this coupling. The whiteness monitor (Section 5A) provides orthogonal diagnostic information but does not directly resolve the Q-R ambiguity. Full resolution requires either: a second independent sensor, structural constraints on the Q/R ratio (e.g., fixed ratio derived from sensor datasheet), or a batch maximum-likelihood Q/R estimation phase before real-time operation.

End of Specification — SignalFix V2.1 Corrected Mathematical Architecture

All subsequent implementation must be traceable to equations defined herein. All 4 critical and 4 high-severity audit findings from the V2.0 review have been addressed. Residual limitations are explicitly documented in Sections 13.4, 13.5, 14.4, and 14.5.